Advanced Graphics – Physically Based Rendering

Jainesh Pathak
June 2023
 MSc Computer Games Engineering
Newcastle University
 J.P.Pathak2@newcastle.ac.uk

ABSTRACT

A video game without lighting is never going to look pretty and will rather look uninteresting. Real world lighting calculation are complex and expensive for video games to handle. Many modern video games make use of a lighting calculation technique called Physically Based Rendering (PBR) which mimics the real-world lighting in a digital world and makes it look realistic. PBR works on microfacet surface model, energy conversations and Bidirectional Reflective Distribution Function (BRDF) to make it happen.

This report explains the process to achieve the approximation of realistic views using PBR and future for the project in context of video game graphics.

KEYWORDS

PBR; BRDF; Lighting; Lights; Irradiance; Billboards; HDR; Tone Mapping; Shading

INTRODUCTION

In computer graphics, lighting plays a vital role of making the player/viewer believe the realism of the digital worlds. In old arcade style games like Pac-Man, Wolfenstein 3D, lighting was non-existent, and player can see the whole level or map even at huge distance (E.g., A corridor in Wolfenstein 3D). Even if expensive lighting algorithms existed, the CPUs of that era were not powerful enough to handle those lighting calculations in real time.

Later, as CPUs and GPUs become more powerful; “close-to” realistic lighting also became possible.

RELATED WORK

- Lighting in old video games

One of the greatest games of all time – Doom and others like Heretic, Hexen did show some sort of dynamic lighting although not realistic which is called Sector-based lighting. In Doom game, the world is divided into sectors which are made by connecting line segments to form a polygon. Every sector had different properties like floor, ceiling, texture and most importantly the light levels. Every sector had a light level ranging from 0-255 with 0 being dark and 255 being very bright.

Lighting attenuation computations were done such that the light travel from one sector to its adjacent sectors and based on the distance of how much it traveled, the light level of current sector will be slowly attenuated. It did support additional light sources like lamp for doing light computation for that sector and its adjacent ones.

  - Light Diminishing

Another fine addition to lighting Doom introduced in “Light Diminishing”. Where the brightness of the area from the player’s point of view slowly decreases as the distance from the player increases. This did not create any realism as well, but it did create impressive scary atmosphere for the game. This same mechanism was also used to simulate fogs. [1]

  - Colormap implementation

Before explaining Colormap, I need to first explain what color quantization is.

    - Color quantization

Quantization in general is an image compression technique which is used to narrow down certain range of values to a single value. This also helps in reducing the file size. Moreover, color quantization works by reducing colors of the image such that the compressed is visually like its original.

In the PC port of original doom engine, it made use a Colormap which is a color quantized gradient texture which starts from a different color starting at top and slowly fades to black color as moving down. It is a precomputed lookup table which was used to fit in the game’s 256-color palette. The first 32 levels in the colormap were purely dedicated for implementing lighting. See Figure 1 for example.

Figure 1: Top – Pure gradient. Bottom – Doom’s quantized colormap light levels

Overall, the lighting calculations in Doom was simple and not realistic, but it did bring immersive gameplay which made the maps look and feel believable.

- Lighting in mid video games

Games from Doom 3, Unreal Tournament onwards started introducing dynamic lighting and shadow casting. Different light sources were introduced like Point Light, Directional Light and Spotlight.

To quickly summarize these different light sources:

  - Point Light:

This type of light source emits lights in all directions. In technical details it contains the position in 3D world, color, intensity value and range.

  - Directional Light:

This light emits light in the single direction, and it has no attenuation and light travel infinitely in the game world. Just like a sun, however the sun in real life is one big point light in the solar system.

  - Spotlight:

This type of light emits light just like directional light, and it falls in a certain radius. Surfaces outside the radius are not lit at all. A good example of spotlight is flashlight which are common to use in horror games.

In real life, when a light falls onto an object, some of it get bounced or reflected and some get absorbed by the surface. The light which gets reflected leaves a white shining effect depending on the object’s shininess. This shining effect is called Specular highlights.

In computer graphics, the most popular shading model for specular highlights in the Phong shading model which he has explained in detail [2]. His work is influenced from previous shading model such that of Warnock’s [3] and Newell, Newell, Sancha’s shading model [4] and Gouraud shading model which needs to be explained first.

  - Warnock’s Shading:

Light source and camera position are placed at the same position and function returns the sum of two terms: Normal and Specular reflection. Surfaces facing towards the light source are always bright compared to its adjacent which were shaded with different light intensities. Also, discontinuity can be easily seen on curved surfaces as seen in Figure 2.

  - Newell, Newell, Sancha’s Shading:

These people showcased an idea for creating specular highlights on transparent object. From experiments in real world, they found out that specular highlights are not created from a light source, but also from other objects as which is highly depends on if the object has high reflective surfaces or transparent like glass. The application of this type of shading unfortunately is highly limited to transparent object and the specular highlights created on glass is like Warnock’s shading as it can be seen in Figure 3.

Figure 2: Warnock’s Shading destroys smoothness on curved surfaces

Figure 3: Newell, Newell, Sancha’s Shading showing highlights on transparent object

  - Gouraud Shading:

Gouraud [5] developed an algorithm which computes the specular highlights on curved surfaces of each vertex of the triangle. From the surface’s curvature, reflection intensity is calculated and is linearly interpolated towards zero between its edges connecting the neighbour surfaces. This generated visually appealing shading effect and retained the smoothness of curved surfaces as it can be seen in Figure 4.

Figure 4: Gouraud Shading effect on low-polygon cone

Unfortunately, the algorithm was still not perfect, and it still created same discontinuity issue if the object has low polygons. If the surface of the object contains high amount of specular reflection, then the highlights are often irregularly shaped because it depends on the shape of the polygons approximate curved surface and not the curved surface of object. In computer generated films, the frame-by-frame discontinuities can be seen even more when the object is in motion, in one frame the faces which are orthogonal to direction of light ray takes a uniform shade and in the next frame, those same faces are in different orientation towards the light source. This same effect happens, when the object’s orientation is changed.

This problem can be solved if the number of polygons is increased, but this is not an efficient solution, because of high memory amount required. Here’s where enters the Phong shading model.

  - Phong Shading:

Bui Tuong Phong [6] introduced new shading model based on the Gouraud shading. It consists of three components viz. Ambient, Diffuse and Specular.

Ambient lighting is where Object is never dark, there is still some amount light in the world. It is a simple constant value that is added to object’s surface color. In simple term, Ambient light is omni-present even when there is not a single light source.

Diffuse lighting is an important which directly simulates the light ray on the object’s surface. Surfaces that face the light source are brighter than the ones that are not facing.

Figure 5: Diffuse Light Ray and surface normal N used to calculate the cosine angle

It is calculated by taking an incident ray or light ray direction vector, the normal vector which perpendicular to the object’s surface and thereby calculating the dot product or the cosine angle of these two vectors. If the angle is greater than zero means the surface of object is facing the light source and vice versa. That is why it is best to check if dot product is always greater than zero beforehand.

To calculate light ray direction is simple by subtracting the light’s position and surface position and normalizing it.

The normal vector is calculated from a triangle of a mesh. It is stored with the mesh data file as it is not required to calculate it every frame. To calculate the normal vector, we first take a difference vector from the two vertices to the one vertex and normalizing it and taking the cross product of those difference vectors gives the normal vector.

Once the angle is retrieved, it is simply multiplied with the light color and lastly added with the ambient component to create a believable image. This can be seen in Figure 6.

Figure 6: Diffuse Lighting on the mesh

Specular lighting computes the bright spot on the surface of the object when it shiny.

For accuracy check of the structured paper, user can run the option Manuscript Validation. It informs the user of the wrong or missing values in the paper. The user must correct the paper as per validation messages and rerun Manuscript Validation.

Now, to generate the required layout of the paper, the user should select one of the template styles under the Define Template Style option (choose the required layout design, i.e. choose between Journals and Proceedings).

Some specific values are required to create a standard layout by choosing a template for the journals or proceedings. So once the user chooses one of the template layout styles, the respective Journal or Conference details dialog box (i.e. journal/conference acronym, DOI, ISBN, copyright, year, etc.) will appear as a prompt during the Define Template Style functionality. The user should fill these values, after which the template creates the desired layout of the paper. The user can now create a PDF of his/her manuscript using the “Save as PDF” option.

If the user is adding any new data, they should make sure to style it as per the instructions provided in previous sections. Carry out the steps for Cross-linking, Fundref data, adding Document History (specific to journal submission), and finally, Manuscript validation and placing the respective metadata (Bibstrip/copyright text) while applying the required template.

CCS CONCEPTS

• Insert CCS text here • Insert CCS text here   • Insert CCS text here

ACM Reference format:

FirstName Surname, FirstName Surname and FirstName Surname. 2018. Insert Your Title Here: Insert Subtitle Here. In Proceedings of ACM Woodstock conference (WOODSTOCK’18). ACM, New York, NY, USA, 2 pages. https://doi.org/10.1145/1234567890

1 Insert Heading Level 1

The updated template, user manuals, samples, and required fonts, all are available at the URL https://www.acm.org/publications/proceedings-template. It contains said information for all three versions of MS Word (Windows and 2 versions of Mac). There are also separate links to the user guide, which can be referred to by the user. This URL also contains some useful video links, which describe how to add the template, structure the paper, and generate the layout, in different clips. Display Formula with Number

(1)

Continuation part of Paragraph Text The user must style this paragraph in ParaContinue style, which follows immediately after the DisplayFormula (numbered equation). The DisplayFormula style is applied only in case of a numbered equation. A numbered equation always has a number to its right. Insert paragraph text here. Display Formula without Number

The DisplayFormulaUnnum style is applied only in case of an unnumbered equation. An unnumbered display equation never contains an equation number to its right, and this unique property distinguishes it from a numbered equation.

Figure 1: Figure Caption and Image above the caption [In draft mode, Image will not appear on the screen]

Theorem/Proof/Lemma. Insert text here for the enunciation or Math statement. Insert text here for the enunciation or Math statement. Insert text here for the enunciation or Math statement. Insert text here for the enunciation or Math statement. Insert text here for the enunciation or Math statement.

....Insert text here for the Quotation or Extract, Insert text here for the Quotation or Extract, Insert text here for the Quotation or Extract, Insert text here for the Quotation or Extract, Insert text here for the Quotation or Extract, Insert text here for the Quotation or Extract.

1.1 Heading Level 2

In the below paragraph, it is explained how alt-txt value is placed in MS Word 2010. To add alternative text to a picture in Word 2010, follow these steps:

- In a Word 2010 document, insert a picture.

- Right click on the inserted picture and select the Format Picture option.

- Select the Alt Txt option from the left-side panel options.

- In the "Title:" and "Description:" text boxes, type the text you want to represent the picture, and then click "Close".

Below are steps to place alt-txt value in MS Word 2013/2016. To add alternative text to a picture in Word 2013/2016, follow these steps:

- In a Word 2013/2016 document, insert a picture.

- Right click on the inserted picture and select the Format Picture option.

- In the settings at the right side of the window, click on the "Layout & Properties" icon (3rd option).

- Expand Alt Txt option.

- In the "Title:" and "Description:" text boxes, type the text you want to represent the picture, and then click "Close".

1.1.1 Heading Level 3. Insert paragraph text here. Insert paragraph text here. Insert paragraph text here. Insert paragraph text here. Insert paragraph text here. Insert paragraph text here. Insert paragraph text here. Insert paragraph text here. Insert paragraph text here. Insert paragraph text here. Insert paragraph text here.

1.1.1.1 Heading Level 4. Insert paragraph text here. Insert paragraph text here. Insert paragraph text here. Insert paragraph text here. Insert paragraph text here. Insert paragraph text here. Insert paragraph text here. Insert paragraph text here. Insert paragraph text here. Insert paragraph text here. Insert paragraph text here.

ACKNOWLEDGMENTS

Insert paragraph text here. Insert paragraph text here. Insert paragraph text here. Insert paragraph text here. Insert paragraph text here. Insert paragraph text here. Insert paragraph text here. Insert paragraph text here. Insert paragraph text here. Insert paragraph text here. Insert paragraph text here.

REFERENCES

[1]	Patricia S. Abril and Robert Plant, 2007. The patent holder's dilemma: Buy, sell, or troll? Commun. ACM 50, 1 (Jan, 2007), 36-44. DOI: https://doi.org/10.1145/1188913.1188915.

[2]	Sten Andler. 1979. Predicate path expressions. In Proceedings of the 6th. ACM SIGACT-SIGPLAN Symposium on Principles of Programming Languages (POPL '79). ACM Press, New York, NY, 226-236. DOI:https://doi.org/10.1145/567752.567774

[3]	Ian Editor (Ed.). 2007. The title of book one (1st. ed.). The name of the series one, Vol. 9. University of Chicago Press, Chicago. DOI:https://doi.org/10.1007/3-540-09237-4.

[4]	David Kosiur. 2001. Understanding Policy-Based Networking (2nd. ed.). Wiley, New York, NY..

Conference Name:ACM Woodstock conference

Conference Short Name:WOODSTOCK’18

Conference Location:El Paso, Texas USA

ISBN:978-1-4503-0000-0/18/06

Year:2018

Date:June

Copyright Year:2018

Copyright Statement:rightsretained

DOI:10.1145/1234567890

RRH: F. Surname et al.

Price:$15.00